A communications satellite is an artificial satellite that relays and amplifies radio telecommunication signals via a transponder; it creates a communication channel between a source transmitter and a receiver at different locations on Earth. Communications satellites are used for television, telephone, radio, internet, and military applications.[1] Some communications satellites are in geostationary orbit 22,236 miles (35,785 km) above the equator, so that the satellite appears stationary at the same point in the sky; therefore the satellite dish antennas of ground stations can be aimed permanently at that spot and do not have to move to track the satellite. However, most form satellite constellations in low Earth orbit, where ground antennas must track the satellites and switch between them frequently.

The radio waves used for telecommunications links travel by line of sight and so are obstructed by the curve of the Earth. The purpose of communications satellites is to relay the signal around the curve of the Earth allowing communication between widely separated geographical points.[2] Communications satellites operate across a wide range of radio and microwave frequencies. To avoid signal interference, international organizations have regulations for which frequency ranges or "bands" certain organizations are allowed to use. This allocation of bands minimizes the risk of signal interference.[3]

## History

### Origins

In October 1945, Arthur C. Clarke published an article titled "Extraterrestrial Relays" in the British magazine Wireless World.[4] The article described the fundamentals behind the deployment of artificial satellites in geostationary orbits to relay radio signals. Because of this, Arthur C. Clarke is often quoted as being the inventor of the concept of the communications satellite, and the term 'Clarke Belt' is employed as a description of the orbit.[5]

Replica of Sputnik 1

The first artificial Earth satellite was Sputnik 1, which was put into orbit by the Soviet Union on 4 October 1957. It was developed by Mikhail Tikhonravov and Sergey Korolev, building on work by Konstantin Tsiolkovsky.[6] Sputnik 1 was equipped with an on-board radio transmitter that worked on two frequencies of 20.005 and 40.002 MHz, or 7 and 15 meters wavelength. The satellite was not placed in orbit to send data from one point on Earth to another, but the radio transmitter was meant to study the properties of radio wave distribution throughout the ionosphere. The launch of Sputnik 1 was a major step in the exploration of space and rocket development, and marks the beginning of the Space Age.[7]

### Early active and passive satellite experiments

There are two major classes of communications satellites, passive and active. Passive satellites only reflect the signal coming from the source, toward the direction of the receiver. With passive satellites, the reflected signal is not amplified at the satellite, and only a small amount of the transmitted energy actually reaches the receiver. Since the satellite is so far above Earth, the radio signal is attenuated due to free-space path loss, so the signal received on Earth is very weak. Active satellites, on the other hand, amplify the received signal before retransmitting it to the receiver on the ground.[3] Passive satellites were the first communications satellites, but are little used now.

Work that was begun in the field of electrical intelligence gathering at the United States Naval Research Laboratory in 1951 led to a project named Communication Moon Relay. Military planners had long shown considerable interest in secure and reliable communications lines as a tactical necessity, and the ultimate goal of this project was the creation of the longest communications circuit in human history, with the Moon, Earth's natural satellite, acting as a passive relay. After achieving the first transoceanic communication between Washington, D.C., and Hawaii on 23 January 1956, this system was publicly inaugurated and put into formal production in January 1960.[8]

The Atlas-B with SCORE on the launch pad; the rocket (without booster engines) constituted the satellite.

The first satellite purpose-built to actively relay communications was Project SCORE, led by Advanced Research Projects Agency (ARPA) and launched on 18 December 1958, which used a tape recorder to carry a stored voice message, as well as to receive, store, and retransmit messages. It was used to send a Christmas greeting to the world from U.S. President Dwight D. Eisenhower. The satellite also executed several realtime transmissions before the non-rechargeable batteries failed on 30 December 1958 after eight hours of actual operation.[9][10]

The direct successor to SCORE was another ARPA-led project called Courier. Courier 1B was launched on 4 October 1960 to explore whether it would be possible to establish a global military communications network by using "delayed repeater" satellites, which receive and store information until commanded to rebroadcast them. After 17 days, a command system failure ended communications from the satellite.[11][12]

NASA's satellite applications program launched the first artificial satellite used for passive relay communications in Echo 1 on 12 August 1960. Echo 1 was an aluminized balloon satellite acting as a passive reflector of microwave signals. Communication signals were bounced off the satellite from one point on Earth to another. This experiment sought to establish the feasibility of worldwide broadcasts of telephone, radio, and television signals.[12][13]

### More firsts and further experiments

Telstar was the first active, direct relay communications commercial satellite and marked the first transatlantic transmission of television signals. Belonging to AT&T as part of a multi-national agreement between AT&T, Bell Telephone Laboratories, NASA, the British General Post Office, and the French National PTT (Post Office) to develop satellite communications, it was launched by NASA from Cape Canaveral on 10 July 1962, in the first privately sponsored space launch.[14][15]

Another passive relay experiment primarily intended for military communications purposes was Project West Ford, which was led by Massachusetts Institute of Technology's Lincoln Laboratory.[16] After an initial failure in 1961, a launch on 9 May 1963 dispersed 350 million copper needle dipoles to create a passive reflecting belt. Even though only about half of the dipoles properly separated from each other,[17] the project was able to successfully experiment and communicate using frequencies in the SHF X band spectrum.[18]

An immediate antecedent of the geostationary satellites was the Hughes Aircraft Company's Syncom 2, launched on 26 July 1963. Syncom 2 was the first communications satellite in a geosynchronous orbit. It revolved around the Earth once per day at constant speed, but because it still had north–south motion, special equipment was needed to track it.[19] Its successor, Syncom 3, launched on 19 July 1964, was the first geostationary communications satellite. Syncom 3 obtained a geosynchronous orbit, without a north–south motion, making it appear from the ground as a stationary object in the sky.[20]

A direct extension of the passive experiments of Project West Ford was the Lincoln Experimental Satellite program, also conducted by the Lincoln Laboratory on behalf of the United States Department of Defense.[16] The LES-1 active communications satellite was launched on 11 February 1965 to explore the feasibility of active solid-state X band long-range military communications. A total of nine satellites were launched between 1965 and 1976 as part of this series.[21][22]

### International commercial satellite projects

A stamp depicting the Palapa D satellite; the Palapa D satellite is a commercial satellite from Indonesia.

In the United States, 1962 saw the creation of the Communications Satellite Corporation (COMSAT) private corporation, which was subject to instruction by the US Government on matters of national policy.[23] Over the next two years, international negotiations led to the Intelsat Agreements, which in turn led to the launch of Intelsat 1, also known as Early Bird, on 6 April 1965, and which was the first commercial communications satellite to be placed in geosynchronous orbit.[24][25] Subsequent Intelsat launches in the 1960s provided multi-destination service and video, audio, and data service to ships at sea (Intelsat 2 in 1966–67), and the completion of a fully global network with Intelsat 3 in 1969–70. By the 1980s, with significant expansions in commercial satellite capacity, Intelsat was on its way to become part of the competitive private telecommunications industry, and had started to get competition from the likes of PanAmSat in the United States, which, ironically, was then bought by its archrival in 2005.[23]

When Intelsat was launched, the United States was the only launch source outside of the Soviet Union, who did not participate in the Intelsat agreements.[23] The Soviet Union launched its first communications satellite on 23 April 1965 as part of the Molniya program.[26] This program was also unique at the time for its use of what then became known as the Molniya orbit, which describes a highly elliptical orbit, with two high apogees daily over the northern hemisphere. This orbit provides a long dwell time over Russian territory as well as over Canada at higher latitudes than geostationary orbits over the equator.[27]

In the 2020s, the popularity of low Earth orbit satellite internet constellations providing relatively low-cost internet services led to reducing demand for new geostationary orbit communications satellites.[28]

## Satellite orbits

Clickable image, highlighting medium altitude orbits around Earth,[a] from Low Earth to the lowest High Earth orbit (geostationary orbit and its graveyard orbit, at one ninth of the Moon's orbital distance),[b] with the Van Allen radiation belts and the Earth to scale

Communications satellites usually have one of three primary types of orbit, while other orbital classifications are used to further specify orbital details. MEO and LEO are non-geostationary orbit (NGSO).

- Geostationary satellites have a geostationary orbit (GEO), which is 22,236 miles (35,785 km) from Earth's surface. This orbit has the characteristic that the satellite’s apparent position in the sky, as viewed from the ground, does not change, the satellite appears to "stand still" in the sky. This is because the satellite's orbital period is the same as the rotation rate of the Earth. The advantage of this orbit is that ground antennas do not have to track the satellite across the sky; they can be fixed to point at the location in the sky the satellite appears.

- Medium Earth orbit (MEO) satellites are closer to Earth. Orbital altitudes range from 2,000 to 36,000 kilometres (1,200 to 22,400 mi) above Earth.

- The region below medium orbits is referred to as low Earth orbit (LEO), and is about 160 to 2,000 kilometres (99 to 1,243 mi) above Earth.

As satellites in MEO and LEO orbit the Earth faster, they do not remain visible in the sky to a fixed point on Earth continually like a geostationary satellite, but appear to a ground observer to cross the sky and "set" when they go behind the Earth beyond the visible horizon. Therefore, to provide continuous communications capability with these lower orbits requires a larger number of satellites, so that one of these satellites will always be visible in the sky for transmission of communication signals. However, due to their closer distance to the Earth, LEO or MEO satellites can communicate to ground with reduced latency and at lower power than would be required from a geosynchronous orbit.[29]

### Low Earth orbit (LEO)

Main article: Low Earth orbit

Low Earth orbit

A low Earth orbit (LEO) typically is a circular orbit about 160 to 2,000 kilometres (99 to 1,243 mi) above the Earth's surface and, correspondingly, a period (time to revolve around the Earth) of about 90 minutes.[30]

Because of their low altitude, these satellites are only visible from within a radius of roughly 1,000 kilometres (620 mi) from the sub-satellite point. In addition, satellites in low Earth orbit change their position relative to the ground position quickly. So even for local applications, many satellites are needed if the mission requires uninterrupted connectivity.

Low-Earth-orbiting satellites are less expensive to launch into orbit than geostationary satellites and, due to proximity to the ground, do not require as high signal strength (signal strength falls off as the square of the distance from the source, so the effect is considerable). Thus there is a trade off between the number of satellites and their cost.

In addition, there are important differences in the onboard and ground equipment needed to support the two types of missions.

### Satellite constellation

Main article: Satellite constellation

A group of satellites working in concert is known as a satellite constellation. Two such constellations, intended to provide satellite phone and low-speed data services, primarily to remote areas, are the Iridium and Globalstar systems. The Iridium system has 66 satellites, which orbital inclination of 86.4° and inter-satellite links provide service availability over the entire surface of Earth. Starlink is a satellite internet constellation operated by SpaceX, that aims for global satellite Internet access coverage.

It is also possible to offer discontinuous coverage using a low-Earth-orbit satellite capable of storing data received while passing over one part of Earth and transmitting it later while passing over another part. This will be the case with the CASCADE system of Canada's CASSIOPE communications satellite. Another system using this store and forward method is Orbcomm.

### Medium Earth orbit (MEO)

Main article: Medium Earth orbit

A medium Earth orbit is a satellite in orbit somewhere between 2,000 and 35,786 kilometres (1,243 and 22,236 mi) above the Earth's surface. MEO satellites are similar to LEO satellites in functionality. MEO satellites are visible for much longer periods of time than LEO satellites, usually between 2 and 8 hours. MEO satellites have a larger coverage area than LEO satellites. A MEO satellite's longer duration of visibility and wider footprint means fewer satellites are needed in a MEO network than a LEO network. One disadvantage is that a MEO satellite's distance gives it a longer time delay and weaker signal than a LEO satellite, although these limitations are not as severe as those of a GEO satellite.

Like LEOs, these satellites do not maintain a stationary distance from the Earth. This is in contrast to the geostationary orbit, where satellites are always 35,786 kilometres (22,236 mi) from Earth.

Typically the orbit of a medium Earth orbit satellite is about 16,000 kilometres (10,000 mi) above Earth.[31] In various patterns, these satellites make the trip around Earth in anywhere from 2 to 8 hours.

#### Examples of MEO

- In 1962, the communications satellite, Telstar, was launched. It was a medium Earth orbit satellite designed to help facilitate high-speed telephone signals. Although it was the first practical way to transmit signals over the horizon, its major drawback was soon realised. Because its orbital period of about 2.5 hours did not match the Earth's rotational period of 24 hours, continuous coverage was impossible. It was apparent that multiple MEOs needed to be used in order to provide continuous coverage.

- In 2013, the first four of a constellation of 20 MEO satellites was launched. The O3b satellites provide broadband internet services, in particular to remote locations and maritime and in-flight use, and orbit at an altitude of 8,063 kilometres (5,010 mi)).[32]

### Geostationary orbit (GEO)

Main article: Geostationary orbit

Geostationary orbit

To an observer on Earth, a satellite in a geostationary orbit appears motionless, in a fixed position in the sky. This is because it revolves around the Earth at Earth's own angular velocity (one revolution per sidereal day, in an equatorial orbit).

A geostationary orbit is useful for communications because ground antennas can be aimed at the satellite without their having to track the satellite's motion. This is relatively inexpensive.

In applications that require many ground antennas, such as DirecTV distribution, the savings in ground equipment can more than outweigh the cost and complexity of placing a satellite into orbit.

#### Examples of GEO

- The first geostationary satellite was Syncom 3, launched on 19 August 1964, and used for communication across the Pacific starting with television coverage of the 1964 Summer Olympics. Shortly after Syncom 3, Intelsat I, aka Early Bird, was launched on 6 April 1965 and placed in orbit at 28° west longitude. It was the first geostationary satellite for telecommunications over the Atlantic Ocean.

- On 9 November 1972, Canada's first geostationary satellite serving the continent, Anik A1, was launched by Telesat Canada, with the United States following suit with the launch of Westar 1 by Western Union on 13 April 1974.

- On 30 May 1974, the first geostationary communications satellite in the world to be three-axis stabilized was launched: the experimental satellite ATS-6 built for NASA.

- After the launches of the Telstar through Westar 1 satellites, RCA Americom (later GE Americom, now SES) launched Satcom 1 in 1975. It was Satcom 1 that was instrumental in helping early cable TV channels such as WTBS (now TBS), HBO, CBN (now Freeform) and The Weather Channel become successful, because these channels distributed their programming to all of the local cable TV headends using the satellite. Additionally, it was the first satellite used by broadcast television networks in the United States, like ABC, NBC, and CBS, to distribute programming to their local affiliate stations. Satcom 1 was widely used because it had twice the communications capacity of the competing Westar 1 in America (24 transponders as opposed to the 12 of Westar 1), resulting in lower transponder-usage costs. Satellites in later decades tended to have even higher transponder numbers.

By 2000, Hughes Space and Communications (now Boeing Satellite Development Center) had built nearly 40 percent of the more than one hundred satellites in service worldwide. Other major satellite manufacturers include Space Systems/Loral, Orbital Sciences Corporation with the Star Bus series, Indian Space Research Organisation, Lockheed Martin (owns the former RCA Astro Electronics/GE Astro Space business), Northrop Grumman, Alcatel Space, now Thales Alenia Space, with the Spacebus series, and Astrium.

### Molniya orbit

Main article: Molniya orbit

Geostationary satellites must operate above the equator and therefore appear lower on the horizon as the receiver gets farther from the equator. This will cause problems for extreme northerly latitudes, affecting connectivity and causing multipath interference (caused by signals reflecting off the ground and into the ground antenna).

Thus, for areas close to the North (and South) Pole, a geostationary satellite may appear below the horizon. Therefore, Molniya orbit satellites have been launched, mainly in Russia, to alleviate this problem.

Molniya orbits can be an appealing alternative in such cases. The Molniya orbit is highly inclined, guaranteeing good elevation over selected positions during the northern portion of the orbit. (Elevation is the extent of the satellite's position above the horizon. Thus, a satellite at the horizon has zero elevation and a satellite directly overhead has elevation of 90 degrees.)

The Molniya orbit is designed so that the satellite spends the great majority of its time over the far northern latitudes, during which its ground footprint moves only slightly. Its period is one half day, so that the satellite is available for operation over the targeted region for six to nine hours every second revolution. In this way a constellation of three Molniya satellites (plus in-orbit spares) can provide uninterrupted coverage.

The first satellite of the Molniya series was launched on 23 April 1965 and was used for experimental transmission of TV signals from a Moscow uplink station to downlink stations located in Siberia and the Russian Far East, in Norilsk, Khabarovsk, Magadan and Vladivostok. In November 1967 Soviet engineers created a unique system of national TV network of satellite television, called Orbita, that was based on Molniya satellites.

### Polar orbit

Main article: Polar orbit

In the United States, the National Polar-orbiting Operational Environmental Satellite System (NPOESS) was established in 1994 to consolidate the polar satellite operations of NASA (National Aeronautics and Space Administration) NOAA (National Oceanic and Atmospheric Administration). NPOESS manages a number of satellites for various purposes; for example, METSAT for meteorological satellite, EUMETSAT for the European branch of the program, and METOP for meteorological operations.

These orbits are Sun synchronous, meaning that they cross the equator at the same local time each day. For example, the satellites in the NPOESS (civilian) orbit will cross the equator, going from south to north, at times 1:30 P.M., 5:30 P.M., and 9:30 P.M.

### Beyond geostationary orbit

There are plans and initiatives to bring dedicated communications satellite beyond geostationary orbits. NASA proposed LunaNet as a data network aiming to provide a "Lunar Internet" for cis-lunar spacecraft and Installations. The Moonlight Initiative is an equivalent ESA project[33][34] that is stated to be compatible and providing navigational services for the lunar surface. Both programmes are satellite constellations of several satellites in various orbits around the Moon.

Other orbits are also planned to be used. Positions in the Earth-Moon-Libration points are also proposed for communication satellites covering the Moon alike communication satellites in geosynchronous orbit cover the Earth.[35][36] Also, dedicated communication satellites in orbits around Mars supporting different missions on surface and other orbits are considered, such as the Mars Telecommunications Orbiter.

## Structure

Communications satellites are usually composed of the following subsystems:

- Communication payload, normally composed of transponders, antennas, amplifiers and switching systems

- Engines used to bring the satellite to its desired orbit

- A station keeping tracking and stabilization subsystem used to keep the satellite in the right orbit, with its antennas pointed in the right direction, and its power system pointed towards the Sun

- Power subsystem, used to power the Satellite systems, normally composed of solar cells, and batteries that maintain power during solar eclipse

- Command and control subsystem, which maintains communications with ground control stations. The ground control Earth stations monitor the satellite performance and control its functionality during various phases of its life-cycle.

The bandwidth available from a satellite depends upon the number of transponders provided by the satellite. Each service (TV, Voice, Internet, radio) requires a different amount of bandwidth for transmission. This is typically known as link budgeting and a network simulator can be used to arrive at the exact value.

## Frequency allocation for satellite systems

Allocating frequencies to satellite services is a complicated process which requires international coordination and planning. This is carried out under the auspices of the International Telecommunication Union (ITU). To facilitate frequency planning, the world is divided into three regions:

- Region 1: Europe, Africa, the Middle East, what was formerly the Soviet Union, and Mongolia

- Region 2: North and South America and Greenland

- Region 3: Asia (excluding region 1 areas), Australia, and the southwest Pacific

Within these regions, frequency bands are allocated to various satellite services, although a given service may be allocated different frequency bands in different regions. Some of the services provided by satellites are:

- Fixed satellite service (FSS)

- Broadcasting satellite service (BSS)

- Mobile-satellite service

- Radionavigation-satellite service

- Meteorological-satellite service

## Applications

### Telephony

Main article: Satellite phone

Satellite phone (Inmarsat) in use in Nias, Indonesia, in April 2005 after the Nias–Simeulue earthquake

The first and historically most important application for communication satellites was in intercontinental long distance telephony. The fixed Public Switched Telephone Network relays telephone calls from land line telephones to an Earth station, where they are then transmitted to a geostationary satellite. The downlink follows an analogous path. Improvements in submarine communications cables through the use of fiber-optics caused some decline in the use of satellites for fixed telephony in the late 20th century.

Satellite communications are still used in many applications today. Remote islands such as Ascension Island, Saint Helena, Diego Garcia, and Easter Island, where no submarine cables are in service, need satellite telephones. There are also regions of some continents and countries where landline telecommunications are rare to non existent, for example large regions of South America, Africa, Canada, China, Russia, and Australia. Satellite communications also provide connection to the edges of Antarctica and Greenland. Other land use for satellite phones are rigs at sea, a backup for hospitals, military, and recreation. Ships at sea, as well as planes, often use satellite phones.[37]

Satellite phone systems can be accomplished by a number of means. On a large scale, often there will be a local telephone system in an isolated area with a link to the telephone system in a main land area. There are also services that will patch a radio signal to a telephone system. In this example, almost any type of satellite can be used. Satellite phones connect directly to a constellation of either geostationary or low-Earth-orbit satellites. Calls are then forwarded to a satellite teleport connected to the Public Switched Telephone Network .

### Television

Main article: Satellite television

As television became the main market, its demand for simultaneous delivery of relatively few signals of large bandwidth to many receivers being a more precise match for the capabilities of geosynchronous comsats. Two satellite types are used for North American television and radio: Direct broadcast satellite (DBS), and Fixed Service Satellite (FSS).

The definitions of FSS and DBS satellites outside of North America, especially in Europe, are a bit more ambiguous. Most satellites used for direct-to-home television in Europe have the same high power output as DBS-class satellites in North America, but use the same linear polarization as FSS-class satellites. Examples of these are the Astra, Eutelsat, and Hotbird spacecraft in orbit over the European continent. Because of this, the terms FSS and DBS are more so used throughout the North American continent, and are uncommon in Europe.

Fixed Service Satellites use the C band, and the lower portions of the Ku band. They are normally used for broadcast feeds to and from television networks and local affiliate stations (such as program feeds for network and syndicated programming, live shots, and backhauls), as well as being used for distance learning by schools and universities, business television (BTV), Videoconferencing, and general commercial telecommunications. FSS satellites are also used to distribute national cable channels to cable television headends.

Free-to-air satellite TV channels are also usually distributed on FSS satellites in the Ku band. The Intelsat Americas 5, Galaxy 10R and AMC 3 satellites over North America provide a quite large amount of FTA channels on their Ku band transponders.

The American Dish Network DBS service has also recently used FSS technology as well for their programming packages requiring their SuperDish antenna, due to Dish Network needing more capacity to carry local television stations per the FCC's "must-carry" regulations, and for more bandwidth to carry HDTV channels.

A direct broadcast satellite is a communications satellite that transmits to small DBS satellite dishes (usually 18 to 24 inches or 45 to 60 cm in diameter). Direct broadcast satellites generally operate in the upper portion of the microwave Ku band. DBS technology is used for DTH-oriented (Direct-To-Home) satellite TV services, such as DirecTV, DISH Network and Orby TV[38] in the United States, Bell Satellite TV and Shaw Direct in Canada, Freesat and Sky in the UK, Ireland, and New Zealand and DSTV in South Africa.

Operating at lower frequency and lower power than DBS, FSS satellites require a much larger dish for reception (3 to 8 feet (1 to 2.5 m) in diameter for Ku band, and 12 feet (3.6 m) or larger for C band). They use linear polarization for each of the transponders' RF input and output (as opposed to circular polarization used by DBS satellites), but this is a minor technical difference that users do not notice. FSS satellite technology was also originally used for DTH satellite TV from the late 1970s to the early 1990s in the United States in the form of TVRO (Television Receive Only) receivers and dishes. It was also used in its Ku band form for the now-defunct Primestar satellite TV service.

Some satellites have been launched that have transponders in the Ka band, such as DirecTV's SPACEWAY-1 satellite, and Anik F2. NASA and ISRO[39][40] have also launched experimental satellites carrying Ka band beacons recently.[41]

Some manufacturers have also introduced special antennas for mobile reception of DBS television. Using Global Positioning System (GPS) technology as a reference, these antennas automatically re-aim to the satellite no matter where or how the vehicle (on which the antenna is mounted) is situated. These mobile satellite antennas are popular with some recreational vehicle owners. Such mobile DBS antennas are also used by JetBlue Airways for DirecTV (supplied by LiveTV, a subsidiary of JetBlue), which passengers can view on-board on LCD screens mounted in the seats.

### Radio broadcasting

Main article: Satellite radio

Satellite radio offers audio broadcast services in some countries, notably the United States. Mobile services allow listeners to roam a continent, listening to the same audio programming anywhere.

A satellite radio or subscription radio (SR) is a digital radio signal that is broadcast by a communications satellite, which covers a much wider geographical range than terrestrial radio signals.

### Amateur radio

Main article: Amateur radio satellite

Amateur radio operators have access to amateur satellites, which have been designed specifically to carry amateur radio traffic. Most such satellites operate as spaceborne repeaters, and are generally accessed by amateurs equipped with UHF or VHF radio equipment and highly directional antennas such as Yagis or dish antennas. Due to launch costs, most current amateur satellites are launched into fairly low Earth orbits, and are designed to deal with only a limited number of brief contacts at any given time. Some satellites also provide data-forwarding services using the X.25 or similar protocols.

### Internet access

Main article: Satellite Internet access

After the 1990s, satellite communication technology has been used as a means to connect to the Internet via broadband data connections. This can be very useful for users who are located in remote areas, and cannot access a broadband connection, or require high availability of services.

### Military

Main article: Military communications satellite

Further information: X Band Satellite Communication

Communications satellites are used for military communications applications, such as Global Command and Control Systems. Examples of military systems that use communication satellites are the MILSTAR, the DSCS, and the FLTSATCOM of the United States, NATO satellites, United Kingdom satellites (for instance Skynet), and satellites of the former Soviet Union. India has launched its first Military Communication satellite GSAT-7, its transponders operate in UHF, F, C and Ku band bands.[42] Typically military satellites operate in the UHF, SHF (also known as X-band) or EHF (also known as Ka band) frequency bands.

As internet access expands, new digital businesses can emerge.

Young entrepreneurs can access online markets, freelance opportunities, and digital education.

Satellite communication therefore contributes not only to connectivity but also to long-term economic development.

## Social Impact of Satellite Communication

Satellite communication has transformed social interaction globally.

Families separated by long distances can communicate more easily through internet-based services.

People can access news and educational content from around the world.

Social media platforms depend heavily on global communication infrastructure.

During emergencies and disasters, satellite communication can save lives.

Relief organizations often rely on portable satellite terminals when normal networks fail.

In remote areas, satellite internet can reduce isolation and improve access to information.

Educational opportunities also improve significantly.

Students can attend online courses and access learning resources that were previously unavailable.

Satellite communication therefore contributes to social inclusion and knowledge sharing.

## Environmental Concerns and Sustainability

Satellite communication has major importance in Africa.

Many rural communities lack reliable terrestrial infrastructure.

Satellites provide internet access, educational resources, healthcare communication, and financial services.

In Ghana, satellite systems support broadcasting, banking, government communication, and rural connectivity.

Educational institutions can use satellite internet to access online learning platforms.

Farmers can receive weather information and agricultural advice.

Telemedicine services can connect rural clinics with specialists in urban hospitals.

Satellite communication also supports national development goals by improving digital inclusion.

## Comparative Analysis: Satellite vs Terrestrial Communication

Satellite and terrestrial communication systems both play important roles.

### Coverage

Satellites provide extremely wide coverage and can reach remote areas.

Terrestrial systems depend on physical infrastructure.

### Latency

Terrestrial systems generally provide lower latency.

GEO satellites experience higher delays.

LEO systems reduce latency significantly.

### Speed

Fiber optic systems often provide higher speeds in urban areas.

However, modern HTS systems are becoming increasingly competitive.

### Cost

Satellite systems require expensive launches and space equipment.

Terrestrial systems require extensive infrastructure installation.

### Deployment

Satellites can provide coverage quickly across large areas.

Fiber deployment takes more time.

### Reliability

Satellites are less vulnerable to ground infrastructure damage.

However, weather can affect signal quality.

## Challenges in Satellite Communication

Despite its advantages, satellite communication faces several challenges.

### Atmospheric Attenuation

Rain, clouds, and atmospheric gases can weaken satellite signals.

This problem is especially severe for Ka-band systems.

### Rain Fade

Rain fade is a major issue in tropical regions.

Heavy rainfall absorbs and scatters radio waves.

Adaptive coding and power control are used to reduce this problem.

### Latency

High latency remains a challenge for GEO satellites.

This affects real-time applications.

### Orbital Debris

The growing number of satellites increases the risk of collisions.

Space debris includes broken satellites and rocket fragments.

International organizations monitor these objects carefully.

### Cybersecurity Threats

Satellite systems are vulnerable to jamming, spoofing, and hacking.

As satellite networks become more connected to the internet, cybersecurity becomes increasingly important.

### High Costs

Launching satellites requires significant investment.

Satellite manufacturing, launch services, insurance, and maintenance are expensive.

Reusable rocket technology is helping reduce costs.

## Future Trends in Satellite Communication

The future of satellite communication is expected to involve major technological transformation.

Several emerging technologies are likely to improve performance, increase accessibility, and reduce operational costs.

### Integration with 6G Networks

Future communication systems are expected to combine terrestrial and satellite networks into one integrated global system.

6G networks may allow devices to switch automatically between terrestrial towers and satellites depending on signal availability.

This could eliminate many current coverage limitations.

### Artificial Intelligence and Automation

Artificial intelligence is already improving satellite operations.

AI systems can analyze network traffic, predict equipment failures, and optimize bandwidth allocation.

Automation reduces operational costs and improves efficiency.

### Internet of Things (IoT)

The Internet of Things involves connecting billions of devices to the internet.

Satellites will play an important role in connecting IoT devices in remote areas.

Agricultural sensors, environmental monitoring systems, shipping containers, and industrial equipment may all depend on satellite connectivity.

### Quantum Communication

Quantum communication represents a possible future breakthrough.

Quantum encryption could make communication far more secure.

Several countries are already experimenting with quantum satellite systems.

### Direct-to-Device Communication

Future satellites may connect directly to ordinary smartphones.

This could allow users to send messages or access emergency services even in areas without mobile towers.

### Expansion of LEO Constellations

LEO constellations are expected to continue growing rapidly.

Companies are launching thousands of satellites to improve global internet access.

These systems may reduce latency and improve broadband performance significantly.

### Smart Satellites

Future satellites may become more autonomous.

Advanced onboard processing could allow satellites to manage communication traffic independently.

Smart satellites may also coordinate with each other more efficiently.

### Reusable Launch Technology

Reusable rockets are reducing launch costs dramatically.

Lower launch costs encourage innovation and increase the number of satellite projects.

This trend may continue to transform the space industry.

### Green Space Technology

Future satellite systems are expected to focus more on sustainability.

Engineers are researching environmentally friendly propulsion systems and recyclable materials.

Reducing space debris will become increasingly important.

## Expanded Comparative Evaluation

A deeper comparison between satellite and terrestrial systems helps highlight the strengths and weaknesses of each technology.

### Infrastructure Requirements

Terrestrial communication requires towers, cables, switching stations, and fiber optic networks.

Building this infrastructure across mountains, forests, deserts, or islands is expensive.

Satellite communication avoids many of these challenges because one satellite can provide wide coverage from space.

### Disaster Resilience

Natural disasters often damage terrestrial infrastructure.

Floods, earthquakes, storms, and fires can destroy communication lines.

Satellites are generally more resilient because the space segment remains unaffected by ground disasters.

Emergency responders often rely on satellite communication during crises.

### Mobility

Satellite systems provide communication for ships, aircraft, and vehicles moving across large areas.

Terrestrial systems have more limited mobility coverage.

### Scalability

Expanding terrestrial networks may require major infrastructure projects.

Satellite systems can expand coverage more quickly.

However, increased satellite traffic may require more orbital resources and spectrum management.

### Quality of Service

Fiber optic systems still provide superior speed and latency in many urban environments.

However, satellite systems continue to improve rapidly.

LEO constellations are narrowing the performance gap.

### Cost Comparison

Satellite user equipment may still be expensive for some households.

However, terrestrial infrastructure installation may be even more expensive in low-density rural areas.

The most effective strategy in many countries is combining both systems.

Hybrid communication networks provide wider coverage and better reliability.

## Expanded Conclusion

Satellite communication has become one of the foundations of the modern digital world.

From television broadcasting and internet connectivity to navigation systems and disaster management, satellites support countless activities that people rely on every day.

The evolution from early communication satellites to advanced High-Throughput Satellites demonstrates how rapidly technology continues to progress.

Innovations such as spot beam systems, beam hopping, phased-array antennas, frequency reuse, and Satellite Zooming have transformed the industry significantly.

These technologies allow satellites to provide faster speeds, higher capacity, and more reliable communication than ever before.

Although challenges such as latency, atmospheric attenuation, cybersecurity risks, orbital congestion, and environmental concerns remain important, engineers and researchers continue developing solutions.

The rise of LEO constellations is reducing latency and improving broadband access.

Artificial intelligence is helping optimize network management and satellite operations.

### Data collection

Near-ground in situ environmental monitoring equipment (such as tide gauges, weather stations, weather buoys, and radiosondes), may use satellites for one-way data transmission or two-way telemetry and telecontrol. It may be based on a secondary payload of a weather satellite (as in the case of GOES and METEOSAT and others in the Argos system) or in dedicated satellites (such as SCD). The data rate is typically much lower than in satellite Internet access.

## References

### Notes

- Orbital periods and speeds are calculated using the relations 4π2R3 = T2GM and V2R = GM, where R is the radius of orbit in metres; T is the orbital period in seconds; V is the orbital speed in m/s; G is the gravitational constant, approximately 6.673×10−11 Nm2/kg2; M is the mass of Earth, approximately 5.98×1024 kg (1.318×1025 lb).

- Approximately 8.6 times when the Moon is nearest (that is, ⁠363,104 km/42,164 km⁠), to 9.6 times when the Moon is farthest (that is, ⁠405,696 km/42,164 km⁠)